Why simulation is key for maintaining fire incident preparedness

Martijn Boosman, Founder & Director of Business Development at E-Semble
Co-Authors: Dr Katherine Lamb, Incident Command Development Facilitator at Oxfordshire Fire & Rescue Service; Ivo Verhoef, Content & Communication Specialist

Ongoing improvements in fire protection and fire safety engineering have drastically reduced the number of fire related injuries and fatalities. That in itself is great news. However, this development has also had an unforeseen effect: because fire incidents are less frequent, fire officers are gaining less operational experience. This increases the risk of skill fade, for both operational and incident command capabilities. Simulation can provide an effective way to tackle this problem.

The decrease in serious fire incidents becomes apparent when you look at the statistics. In England, for instance, the number of fire fatalities in 2013-2014 was 39% lower than in 2003-2004.And the rate of hospitalized, non-fatal fire casualties dropped 55% between 2003 and 2013. In the same period, fire and rescue authorities attended 48% less fire incidents, and 24% less road traffic collisions.1, 2

As result of this development, Fire officers get less chance to test and develop their skills in the field. On average, fire service commanders do not experience a major fire until their third year in command. This increases the risk of skill fade, for both operational and incident command capabilities. Incident command simulation (not to be confused with fluid dynamics fire and smoke simulation) can provide an effective way to tackle this problem.

Simulation is nothing new

Of course, simulation – or as the definition says: ‘the [purposeful] imitation of the operation of a real world process or system over time’3 – has already been in use to train emergency services personnel for decades. The value of simulation for training purposes lies in its ability to have trainees experience incident situations in a safe, contained, comfortable, repeatable, controllable, and measurable environment.

Within the UK Fire & Rescue Services, simulation is routinely used to during the development of incident commanders, as it provides an appropriate tool to train the delegates during technical and practical skill acquisition, and practical application of these new competencies4.

In particular, it can be used to

1) Help trainees understand command and control decision making concepts by experience,

2) Acquire practical knowledge in a relatively short timeframe,

3) Practice decision making in critical situations and

4) Experience situations that rarely occur in real life.

Originally, most applications of simulation in training emergency personnel were so-called live simulations. This means building a physical environment in which an incident scenario is played out to create incident scenarios. In the late 1990’s, virtual reality simulations reached a level of sophistication that made them suitable for emergency services training. Rapid developments in game technology stimulated the application of virtual reality simulation for non-entertainment purposes.

Benefits of virtual reality

Computer simulations like “computational fluid dynamics” simulators are aimed at accurately representing calculated physics-based processes. Virtual reality simulations are 3D virtual environments in which the user can move around whilst being confronted with a real-time visualisation of all visual and auditory aspects surrounding an incident. Having to be “real-time”, 3D virtual reality simulations have a lower realism (or fidelity) than before mentioned computer simulations. ‘virtual reality’ simulations.

Virtual reality simulations are generally used for training under circumstances that would be too costly, too complex or too dangerous to recreate in real life. The main benefits of computer simulation compared to live simulation are cost effectiveness, flexibility and the ability to create large scale, complicated and repeatable events. Virtual reality is cost effective, because it requires less training staff and eliminates the need to build physical training environments. It is flexible, because instructors can easily build an enormous variety of incident scenarios by picking and combining different environments and objects from a database. But most importantly, modern simulation software allows instructors to create events in a large virtual environment that would be extremely hard to recreate in live simulation – for instance a large motorway accidents or a rapidly spreading fire in a chemical plant. An additional benefit is the ability to replay scenarios for evaluation purposes, for which you would need to set up dozens of cameras in a live simulation setting.

State-of-the-art training centres

In June 2014, the first of two state-of-the-art training centres opened in London, one in Beckton (East London) and one in Harrow (West London). The new centres are part of a 25-year contract between London Fire Brigade and engineering support services provider Babcock International. The facilities feature a 3,200 square meter indoor centre, which include both a training building in which crews can experience live (hot) fire and  a purpose-built Urban Search and Rescue training building which allows crews to train in a wide variety of different rescue operations. One of the facility’s stand-out features is the extensive application of virtual reality.

The main application of virtual reality is the Incident Command Simulation Suite, which consists of four ‘pods’, small rooms featuring a large, floor to ceiling video screen, audio and fully functional replicas of standard issue London Fire Brigade communication equipment. During team exercises, the trainees in the pods each experience a joint simulation scenario from their own point of view. To achieve additional immersion, senior fire officers can train in an exact replica of a London Fire Brigade Incident Command Unit, in which simulation images generated can be displayed on a large monitor.

Virtual reality exercise support can also be combined with live exercises running in the indoor training centres to create a “hybrid training” approach. In hybrid training, the commanding officers have to both command crews operating in the real training area as well as virtual crews operating in in one or more virtual incidents. The advantage of such hybrid approach lies in the fact that the size of the scenario and as result, the span of control for the participants can go beyond the limits of the live training location by simply adding one or more virtual incident locations.

Developing decision making skills

Virtual reality will never fully replace real life simulation, nor should it. However, if the scenarios created are realistic, achievable and manageable given a candidate’s current level of command skill, expertise, and - importantly - be appropriate to their role, then the maximum benefit can be achieved. The same skills needed to resolve both simulated and actual incidents, including good communication skills, effective use of resources and information, and a clear command strategy, are all essential during the dynamic response to an incident.

Incident Command Development facilitators, Dr Katherine Lamb and Jim Davies have registered remarkable results in the assessment of incident command competencies, by integrating virtual reality simulation into a training and assessment methodology of their creation, called the Introspect model4. In their 2014 research paper ‘Incident Command Training: The Introspect Model’ it is argued that virtual reality simulation provides an effective framework to develop these cognitive decision making skills in incident commander.

In this framework, candidates are assessed on their ability to make the ‘right’ dynamic decision based on their own knowledge and evaluation of the incident rather than learn how to apply a scripted list of incident decisions which may not fit the particular incident they are faced with. It establishes a level of assurance by providing a robust assessment & training methodology, which ensures that organisations are able to appoint, train & assess their incident commanders, comfortable in their knowledge that during crisis situations: ‘They have the right person making the right decisions, at the right time, for the right reasons’. To date, this process has been adopted as best practise by many UK FRS, and through published research, this is a proven programme which has improved operational competency & assurance within UK Fire & Rescue Services.

The merits of fidelity

But what makes an effective virtual reality simulation? A common focus point is fidelity – how faithful and accurate is the simulation in its reproduction of reality? Increased fidelity can be achieved in several ways, for instance by improving visual projection technology. In recent years, interesting developments in this area have been the introduction of HD-video, and virtual reality goggles such as the Oculus Rift. Another way of achieving fidelity is to utilise the increased processing power of computer hardware to create more realistic behaving virtual scenarios. For instance, by using computational fluid dynamics (CFD) to simulate lifelike behaviour of smoke, or building virtual replicas of real geographical environments by using geospatial (GIS) data. These areas in which existing Fire Protection and Fire Safety Engineering can add a lot of value. The available research and simulation models can be used to increase the fidelity of virtual reality simulation. A good example of the latter is the International Wildfire Simulation Project, initiated by the ECASC (Ecole d’Application de Securite Civile) College in the South of France

While it might seem attractive to implement these innovations as soon as possible in all simulation based training programmes, one should always remain critical and consider whether a new technology actually leads to a more effective training tool. A good example is the development of an advanced wildfire training simulator for ECASC (Ecole d’Application de Securite Civile) Civil Defence College in the South of France. The original idea to create this simulator was to combine the existing XVR simulation platform5 with the existing wildfire behaviour simulator FARSITE6.  From a realism point of view the combination was ideal where it not for the fact that FARSITE lacked a key element required in training: the real-time response of the fire in response to sudden decision by the student (like more water, or retardant drops). After due consideration, the decision was taken to create a simplified “for training only” computer simulation “Ignis” which provides enough realism for training but should not be used for operational preparedness. Maximizing fidelity is only part of the most important objective for simulation based training: achieving immersion for the participants. And immersion – experiencing a sense of realism and involvement in a simulation – depends in a large part on the role of the instructor.

Creativity, gut feeling, and experience

A common saying among emergency services instructors is: ‘there is no single truth in incident command’. In other words, there is often more than one solution to a problem.  In response to operational situations, incident commanders rely on recognised prime decision making, or pattern matching to resolve an incident7. The experiences of the incident commander from both practical incident exposure, training events using simulation or technical knowledge of operating procedures, are all utilised during the dynamic decision  making process. The incident commander tests and discards each option instantaneously, before establishing the ‘best fit’ or the most plausible solution to resolve the incident. The use of simulation provides the incident commander to limitless opportunities to build experience, and train their recognition primed decision making process to be able to effectively use creativity and intuition in order to make split-second decisions under pressure.

Virtual reality simulation offers excellent opportunities to focus on all these elements, provided that the instructor can use the simulation effectively to guide the students through an incident scenario. Simulation can be highly realistic, when you use well-developed scenarios. By putting the trainees in an interactive environment where their actions directly influence the outcome of events, they are confronted with all the pressures of an actual incident. This creates a powerful immersive effect, which stimulates trainees to use their creativity and gut feeling to make decisions in an incident scenario. Because trainees actually experience the scenario rather than learning the theory, simulation is demonstrably better in training Incident Command Decision Making than classroom based teaching. 8

Off-the-shelf, web-based simulation

It is likely that the ongoing developments in fire engineering will lead to a continuing decrease of operational fire incidents in the future. This also means that maintaining sufficient operational experience and incident preparedness among fire officers will become an increasingly urgent issue. One effective way to tackle this issue is to improve the accessibility of simulation technology. In the coming years, increasing availability of web-based, off-the-shelf simulation software, can meet the need for flexible, cost-effective training solutions. With an continuous increase in computer power it will also be key to combine virtual simulation training with the ongoing improvements in fire protection and fire safety simulation to achieve the highest possible fidelity for the students.

Tackling the issue of skill fade

Rather than one type of simulation replacing another, real life and virtual reality simulation will continue to be used side by side for training fire officers and other incident responders.  They are complementary disciplines that each offer specific benefits for training specific skills and capabilities. Having said that, virtual reality has proven itself as a particularly useful tool to counter the increasing risk of skill fade among incident commanders. Its ability to let trainees experience and act in large scale, complex events, as well as its flexibility and cost effectiveness, makes virtual reality a crucial tool to achieve and maintain sufficient fire incident preparedness. Although fire behaviour and fire safety engineers may have caused the problem of skill fade by making the world a safer place, they also have an important role in solving the problem by helping create high fidelity virtual simulation training tools.

Sources

1 Fire Statistics Monitor: England April 2013 to March 2014, Department for Communities and Local Government.

2 Facing the Future: Findings from the review of efficiencies and operations in fire and rescue authorities in England, Sir Ken Knight CBE QFSM FIFireE (2013).

3 Oxford English Dictionary.

4 Incident Command Training: The Introspect Model. International Journal of Emergency Services.

Lamb, K. J., Davies, J., Bowley, R., Williams, J-P. (2014), Vol. 3, no. 2, pp131-143.

5 www.xvrsim.com

6 http://www.firelab.org/project/farsite

7 Naturalistic decision making. Human Factors. Klein G. A., (2008), Vol. 50, no. 3, pp456-460.

8 HALL, Kurt - Effect Sim on Incident Command Trf Dissertation--3414910